Manufacturing Engineering

ASSIGNMENT 1: TECHNICAL DESCRIPTION

Injection Molding Machine FT Series

Siti Raihana Binti Shahadan

FA08012

Faculty of Manufacturing Engineering

University Malaysia Pahang

TECHNICAL DESCRIPTION: INJECTION MOLDING MACHINE FT SERIES

Injection molding machine is equipment that undergo manufacturing process in producing molded product from plastic material. Injection molding machine FT series is divided into two units, clamping unit and injection unit. This description covers about the major components in this machine and its function, the special features and the procedures to operate this machine.

                     
                     Figure 1.1: Injection Molding machine FT series

           Injection molding machine FT series consists of a few unique major components that are screw, nozzle and nozzle tip, mold and etc. Firstly, screw also known as reciprocating screw is made up of injection molding screw and screw tip assembly. The purposes of this component are to convey material to the front of the injection unit and inject the plastic under high pressure into the mold. Next is nozzle and nozzle tip. These two components are placed at the exit point for the plastic resin from the screw and barrel to the mold. Lastly, the mold consists of injection mold and ejector mold. It size and design depend on the part needed to produce.

Figure 1.2: The major components in Injection molding machine FT series

Furthermore, this model also comes with user-friendly microprocessor controller with a few additional features such as dust, oil proof and colourful touch switch control panel. Besides, this FT Series also equipped with optional items that are close and double loop system for the high precision and stability and shorten the production cycle time.

There are two main procedures to operate this machine, injection molding start-up and injection molding shutdown. Before undergo the process, firstly open the main water valve and close the by pass valve. Then, switch on the main electrical switch. Press and release the controller start key before switch on the pump. Wait until soaking time elapsed and the desired temperature achieved. Next, open the water regulator to machine section and lastly, run the machine with dry cycle mode. After we had done the operation, close the water regulator. Before shutting off the machine, coat the mold with anti-rust and close the mold half way to prevent condensation. Then, switch off the motor, main power supply, main water supply and open the by pass valve. Lastly, do some housekeeping.

As a conclusion, this machine has all the needed components to enhance its performance. Besides, it is outfitted with all the necessary additional functions, auxiliary equipment and mold to adapt as maximum as possible to a specific production process and obtain high quality product.

References

1. A Basic Plastic Injection Molding Process.Retrieved January 11, 2010 from

http://www.wm8c.com.

2. Injection Molding Department. (2009). Higher Institute for Plastics Fabrication :

Workbook for Injection Molding Practical Course. Retrieved January 19,

2010 from http://www.hipf.edu.sa.